Data Quality
Introduction to Quality Assurance

| Project Name | DQ MODULE | Project code | EPM-DQ |
| --- | --- | --- | --- |
| Object Name | Vending Machine Necta Orchestra | QA Engineer | Volha Bolatava |
| Classification by access | Black box – without possibility to explore programing aspects, object components separately or take object apart |  |  |
| Object description | Overview: - Brand: Necta - Height: 1830 mm - Width: 900 mm - Depth: 830 mm - Weight: 300 kg - Voltage: 230 V - Frequency: 50 Hz Capacity & Layout: - Default trays: 6 (expandable up to 8) - Total product capacity: 438 items - Supported configurations: Snacks + Food + Cans & Bottles / Snacks + Food / Food only / Cans & Bottles only Features: - Numeric selection keypad (up to 46 unique product selections) - Softvend elevator dispensing system - gently delivers fragile items without dropping - Stratified temperature layout - separate cooling zones for different product types Model: Vending Machine Necta Orchestra Necta Orchestra ¦ Vendmarque Requirements: - Must support all listed product configurations (snacks, fresh food, cans, bottles) - Dispensing system must handle delicate items safely (no dropping or shaking) - Temperature zones must maintain appropriate preservation conditions per product type - Payment system integration via Executive, MDB, and BDV protocols - Keypad must support minimum 46 selectable products |  |  |

SMOKE PATH

| Verification area | Test Approach by way of dealing | Would be automated? | Test Type | Expected Result / Comments |
| --- | --- | --- | --- | --- |
| Machine powers on and welcome screen displays | Positive | No | Functional | Display shows welcome screen with no errors or flickering |
| Numeric keypad backlight on power-up | Positive | No | Functional | Backlight activates within 3 seconds of power-on |
| Electronic display clarity (characters, prices, error codes) | Positive | No | Functional | All characters are fully visible on display |
| Coin recognition and balance update | Positive | No | Functional | Inserted coin is accepted and balance updates immediately on screen |
| Product slot selection and info display | Positive | No | Functional | Machine highlights selected slot and shows correct product name and price |
| Product dispensing after sufficient payment | Positive | No | Functional | Softvend elevator delivers product to collection tray without dropping or damage |
| No dispense when no money inserted | Negative | No | Functional | Machine shows "insufficient funds"; no product is released |
| Invalid slot number entry | Negative | No | Functional | Machine rejects unrecognised input and prompts re-entry without resetting balance |

CRITICAL PATH

| Verification area | Test Approach by way of dealing | Would be automated? | Test Type | Expected Result / Comments |
| --- | --- | --- | --- | --- |
| Purchase with banknote + change returned | Positive | No | Functional | Insert 5 EUR, select 1.50 EUR item — 3.50 EUR change dispensed correctly |
| Purchase via contactless bank card | Positive | No | Functional | Card tap approved on screen; product dispensed without delay |
| Purchase via mobile payment (Google Pay / Apple Pay) | Positive | No | Functional | NFC transaction confirmed; item delivered to collection tray |
| Exact change payment — no change returned | Positive | No | Functional | Coins equal product price exactly; product dispensed, zero change returned |
| Coin dispenser out of change | Negative | No | Functional | Cash payment disabled; appropriate message shown |
| Payment response time (card/NFC) | Positive | Yes | Performance | Transaction approved and product dispensed within acceptable time threshold |
| Softvend elevator — snack item from upper tray | Positive | No | Functional | Elevator rises smoothly to correct tray level and delivers item intact |
| Numeric keypad — all 46 product codes | Positive | No | Functional | Each valid code displays correct product name, price, and availability |
| Elevator travel speed and smoothness | Positive | Yes | Performance | Elevator completes full tray-to-bin cycle within expected time; no jerking |
| Protocol integration — MDB, Executive, BDV | Positive | Yes | Compatibility | All three communication protocols connect and exchange data without errors |
| Long-term cooling zone stability (8-hour test) | Positive | Yes | Reliability | Temperature zones maintain correct ranges consistently over full shift |
| Product stuck mid-dispense — jam detection | Negative | No | Functional | Machine detects jam, cancels transaction, and refunds full payment |

EXTENDED PATH

| Verification area | Test Approach by way of dealing | Would be automated? | Test Type | Expected Result / Comments |
| --- | --- | --- | --- | --- |
| Foreign coin insertion (e.g. US quarter) | Negative | No | Functional | Coin rejected and returned; balance not credited |
| Bent/damaged coin insertion | Negative | No | Functional | Machine rejects coin without jamming; prompts retry |
| Keypad input with gloves on | Positive | No | Usability | All key presses register correctly through standard glove material |
| Screen visibility in bright sunlight | Positive | No | Usability | Display content fully legible through glass wall under direct sunlight |
| Service door opened without key/code | Negative | No | Safety | Anti-theft alarm activates; alert sent to operator immediately |
| Machine forcefully shaken — tilt sensor | Negative | No | Safety | Tilt sensor triggers; machine locks and suspends all operations |
| Language switch — full UI translation | Positive | No | Usability | All on-screen text updates to selected language; no missing or untranslated strings |
| Anti-theft alarm reset by operator | Positive | No | Functional | Operator can disarm alarm with valid key/code; machine resumes normal operation |

The checklist defines a structured testing approach for the Necta Orchestra vending machine. It classifies test cases into functional and non-functional categories, covering the full user journey from power-on and product selection through payment processing, dispensing, and error recovery. Tests are differentiated by positive and negative scenarios, as well as by their potential for automation. Together, the checklist ensures that the machine's core operations, payment reliability, mechanical dispensing, and environmental performance are thoroughly verified before deployment. This classification is valuable because it demonstrates coverage across priorities and risks, ensuring both essential functionality and extended quality requirements are addressed.

Types of testing used in the checklist:

- Functional testing – verifies that the machine performs actions according to specifications.

- Performance testing – evaluates speed, efficiency, and environmental resilience.

- Safety testing – checks protective features and anti-theft mechanism.

- Usability testing – ensures intuitive and accessible operation for end users .

- Reliability testing – confirms consistent behavior over repeated and extended use.

- Compatibility testing – verifies interoperability with external systems and payment standards.